Code Overview

# 1. Projektstruktur

## C-Project (see Section 2 and 3.2)

- Source code of the C++ project running on the ActivityBots

- uses the Library PopWare

- Follows the lines on the floor and waits for commands from the PC

## Routing Table (see Section 2 and 3.1)

- Source Code of the Kotlin/Java Project running on the PC

- Manages communication between Bots and PC

- Executes the Algorithm and sends instructions to the individual Bots

## Barcode Encoder

- Helper project to automatically generate printable roundabout segments

- Encodes the given node ID into a barcode and saves it into a svg or pdf file

- InkScape must be installed for the conversion from svg to pdf

## Ant Shield

- 3d-printable shield for improving the performance of the LineFollower

- Blocks incoming light from the sides and therefore improves the detection rate of the black lines

- Available as Creo Part, Step and STL

## Printables

- This folder contains all printable rail-segments that make up the map the Ants drive on

- The segments have to printed on A3 Paper, cut to size, optionally they can be laminated to improve longevity

- Additionally, a printable calibration strip is given to check how well the LineFollower is working

- The individual sensors should only kick on in the center of the strips, otherwise it has to be recalibrated

- A representation of the map shown in the demo video can be found in the file 2013Map.xlsx

## Documentation

- Further documentation

- Overview of the code with project structure as well as relevant code segments

- Assembly instructions of the ActivityBots hardware

# 2. System Overview

Routing Table (Kotlin)

Communication over TCP with own Messages-Framing

- Wifi-Handler in ThreadPool

- Encoding and decoding of messages

- Automatic reconnect

- Timeout of Ants using regular pings

Graphical interface

- Chart

- Graph

- Speed-Control

GUI

Communication

Data

Real Ant (C++)

Line-Follower

Communication

- Wifi-Connectivity using external ESP8266

- Encoding and decoding of messages

- Automatic answer to ping-requests

- Line detection using infrared-diodes (B/W)

- Line keeping using PI-Controller

- Collision avoidance using Ultrasonic distance sensor

- Reading and decoding the barcodes

- Real + virtual ants

- ACO-Algorithm

- Representation of the graphs

3. Relevant code segments for changes

## 3.1 Routing Table – Server (Kotlin)

- Connection Settings (IP-Space of the Bots)

  - de.jonasnick.antnet.routingtable.communication.ConnectionManager

    - prefix

- Predefined constants of the Algorithm

  - de.jonasnick.antnet.routingtable.data.AntConstants

- Predefined map

  - de.jonasnick.antnet.routingtable.data.map.Maps

- Function for setting the pheromones on target arrival

  - de.jonasnick.antnet.routingtable.data.map.Connection

    - copyWithAdjustedPheromones

- Function for pheromone evaporation in each tick

  - de.jonasnick.antnet.routingtable.data.AntManager

    - tick -> “nodeManager.updateAllConnections”

- Function for selecting the next edge to follow

  - de.jonasnick.antnet.routingtable.data.ants.AbstractAnt

    - selectNext

## 3.2 ActivityBot – Client (C++)

- Parameter of the PI controller

  - src\programms\linefollower\linefollower.h

    - LineFollower::KID

  - src\programms\linefollower\linefollower.cpp

    - LineFollower::KP

- Min distance of the ultrasonic sensors

  - src\programms\linefollower\linefollower.h

    - MIN_DISTANCE

- Controller for line keeping

  - src\programms\linefollower\linefollower.cpp

    - LineFollower::run (Zeilen 135-200)

- Processing of a read barcode

  - src\programms\linefollower\linefollower.cpp

    - LineFollower::barcodeCallback

- Reading of a barcode

  - src\programms\linefollower\linefollower.cpp

    - LineFollower::run (Zeilen 240-300)

# 4. ActivityBot + PropWare

Propware and this project can be executed on the ActivityBots using the the following instructions:

- Installation of PropWare using the following instructions
https://david.zemon.name/PropWare/#/download

- Installation of MinGW, to be able to use make 
http://www.mingw.org/

- When using Jetbrains CLion: Configuration of the Toolchain as shown:

- Creation of a /bin directory and changing into it

  - mkdir bin

  - cd bin

- Execution of the command (only has to be executed again on changes to CMakeList.txt)

  - cmake -G "Unix Makefiles" ..

- Connect an Activity Bots and set the switch it to Mode 1

- Execute one these commands (still in /bin directory)

  - make debug
To flash the bots in the RAM. 
Program doesn’t persist after switching the bot on and off

  - make debug-eeprom
To flash the bots in the EEPROM.
Program persists even after shutdown or reset